University of Macau

Institute of Artificial Intelligence and Brain Science

CISC7298 Project Report

Project Title and Description

1st Semester 2026/2027

| Project Title (Can be updated later) | Scenario-Adaptive Hierarchical AI for Lifelong Multi-Agent Pickup and Delivery |
| --- | --- |
| Project Description | What is the current problem? Lifelong multi-agent pickup and delivery (MAPD) in warehouses is not one fixed problem: traffic and task patterns change over time. At least three recurring situations appear in practice: (1) Easy / diverse traffic — tasks are spread across many pickups and dropoffs; a lightweight spacetime-A* baseline is already fast and safe. (2) Hotspot batches — many tasks share the same dropoff and/or pickup; greedy baselines congest at the bay, and oversized joint planning often fails or stalls. (3) Complex / high-obstacle maps — narrow corridors make large simultaneous joint windows infeasible, so the system must shrink the planning set and change route style. A single static algorithm cannot fit all three. Using a heavy solver everywhere wastes time on easy scenes; using only greedy A* everywhere collapses under hotspots or hard maps. The research gap is therefore not “one more planner,” but how to recognize the situation and dispatch the right method stack. What do you wish to solve? This project studies scenario-adaptive hierarchical AI for lifelong MAPD: an upper-level AI classifies the current operating regime and selects which algorithm path to run; lower-level AIs then perform situation-specific optimization (who enters the wave, whether to swap tasks, how to share corridors). Easy scenes should keep near-baseline speed; hard or hotspot scenes should escalate only when needed and recover to the fast path when traffic returns to normal. How will you solve it? We organize the system around scenarios rather than a flat module list. Scenario recognition (upper AI — EscalateNet). Each planning wave, the upper model reads map hardness, pickup/dropoff concentration, backlog shape, and recent planning failures. It decides: stay on the fast baseline path, or open “hard-wave” mode. Hysteresis returns to the fast path after consecutive calm waves, so lifelong runs can switch regimes as tasks change. Situation-specific lower AIs (only after escalation, or when the scenario warrants them). • Wave selection (WaveNet): under hotspot or hard-map regimes, choose which AGVs enter the current joint window (bounded concurrency) instead of planning everyone together. • Task swap (SwapNet): when station-level assignment is inverted (far vehicle vs nearer idle), decide whether to steal/swap—useful mainly in congested assign regimes, not on every easy tick. • Traffic / follow acceleration (TrafficNet-lite): when many agents share a destination, prefer a shared outer-ring corridor and leader→follower ordering to cut head-on and idle sync waste. Execution layer. Selected agents are planned with reservation-aware spacetime search / windowed joint planning; trajectories are validated for collisions, illegal motion, and FIFO. Evaluation by scenario. We compare methods under controlled task situations (diverse vs same-dropoff vs same-pickup vs both) and under open vs obstacle-rich maps, reporting completion, sim/wall time, VALID checks, and how often the upper AI escalates or recovers. What do you expect the outcome to be? (1) A clear scenario taxonomy for lifelong MAPD and a reproducible benchmark covering those cases. (2) An upper AI that selects algorithm regimes by situation, preserving fast behavior on easy/diverse traffic. (3) Lower AIs that specialize per regime (wave membership, swap, corridor follow) rather than one end-to-end policy for everything. (4) Empirical evidence that hotspot and hard-map scenes need escalation, while diverse easy scenes should not pay that cost. (5) Validated trajectories and quantitative A/B reports against non-adaptive baselines. |